- System 21

- Binary Software

- User’s Manual

Table Of Contents

1.	Scope	3

1.1	Definitions	3

2.	Revision History	4

3.	Related Documents	4

4.	Overview	5

5.	IEC-1131	6

5.1	Architecture	6

5.1.1	Hierarchical Naming Conventions	7

5.2	EGS Implementation	8

5.2.1	Libraries	8

5.2.2	Projects	9

6.	Application Utilities	12

6.1	PC Based	12

6.1.1	Kermit	12

6.1.2	HyperTerminal	14

6.1.3	RexTerm	16

6.1.4	Router	16

6.1.5	UDP Server	17

6.1.6	NFP Server	17

6.2	iBox Based	17

6.2.1	The X-Shell	17

6.2.2	resinst	21

6.2.3	resvrf	23

6.2.4	gfstool	24

6.2.5	Firmware Tools	Error! Bookmark not defined.

Appendix A – Sample resinst session	Error! Bookmark not defined.

# Scope

This document is intended as an aid in using and understanding the software tools and utilities available for the iBox. It is also intended as a information source for System 21 Training.

## Definitions

| Beta Sensor |  |
| --- | --- |
| FSIR Sensor |  |
| GCT |  |
| HyperTerminal |  |
| Kermit |  |
| iBox |  |

# Revision History

| Date | Version | Author(s) | Comments |
| --- | --- | --- | --- |
| 01-Aug-02 | 1.0 | Bill Sanzo | Initial Version |

# Related Documents

|  | Title | Document Number | Revision and date |
| --- | --- | --- | --- |
| [1] | HOBOKEN Functional Specification | HP391617 | 1.4 - 16Nov95 |

# Overview

The binary loaded to an iBox contains the Resource (which is all the application code written with GCT for the particular iBox); the Resource Manager (which is the interface software that connects the resource to the operating system along with some utilities); the Board Support software (which is the software that interfaces to the hardware, e.g. Drivers); and the ROOT software which is the startup code that is called by the firmware when the iBox is loaded.

The Resource Manager includes a user interface (the X-Shell) which supports a list of diagnostic and utility commands.  The X-Shell allows useful commands such as help, dir, copy, etc., as well as an environment for executing utility programs that were created specifically for the resource.

The X-Shell can be interfaced with through the terminal serial port on the iBox, or by using the CMS terminal emulator program (rexterm) over the Ethernet from a PC. Connecting serially to the iBox can be done with an actual terminal or, more likely, a terminal emulation program like Kermit or HyperTerminal from a PC.

One of the most useful utility programs available is resinst. It allows reading and/or writing any variable in the resource. The hierarchical variable naming convention used by resinst (and the other utilities) is also used to create entries in the MDB file used for the OPC Server. This convention is also used in the AIF, RIF, VRF and RLW files loaded to the iBox automatically after startup.

# IEC-1131

IEC-1131 is a specification for PLC programming languages produced by the International ElectroTechnical Commission. The purpose of IEC-1131 was to create a common set of software languages that could be implemented by various PLC vendors with the goal of simplified learning by the end users. The Graphical Configuration Tool (GCT) was created by Eurotherm Controls as a specific implementation of the IEC-1131 specification.  Wonderware’s InControl is also based on IEC-1131.

The languages included in the IEC-1131 spec include Structured Text (ST), Function Block Diagrams (FBD), Sequential Function Charts (SFC) and Ladder Logic. EGS, using GCT, supports ST, FBD and SFC.

## Architecture

The IEC-1131 specification defines several levels of a hierarchical architecture.

Function Blocks are the most common unit. A function block contains data, instructions, other function blocks and/or functions.  Function blocks have inputs, outputs and/or internal variables. The outputs of one function block can be directly “wired” to the inputs of another block on the same level of the hierarchy, provided the data is of the same form.

Data can be defined in many different forms (BOOL, INT, REAL, etc.). Data can also be represented by Structures, which is a predefined collection of data (e.g., the calibration data for a Beta Sensor). GCT implements the structure concept using User Defined data type (e.g., BetaCalib). GCT allows data to be defined using ST or FBD.  Function Block Diagrams are the preferred method of defining data because they are graphical, they allow data to be wired together, and they support getting to the definition of a lower level function block simply by double clicking the icon for the lower level block in the current function block.  Internally, GCT keeps all the function block and data definitions in structured text, but displays it for the user in the requested format. So data can be defined directly in ST, but the only noteworthy advantage is the ability to comment individual lines of code.

Instructions are typically written directly in ST because it allows simple algebraic expressions, logical compares, IF-THEN-ELSE constructs and direct calls of lower level function blocks. ST is a Basic-like language which is fairly intuitive and very easy to read. Writing code is simple once the formats and keywords are understood.

Instructions can also be implemented using Sequential Function Charts (SFCs). SFCs break the implementation of code in a Function Block into separate states, with transitions between the states, and ST code that is implemented while in a particular state. The only implementation of an SFC in current EGS applications is in the Frame Program. Different ST code is executed depending on whether the SFC is in SCAN, OFFSHEET, STDZ, etc.

Functions are simple, reusable code fragments, like subroutines. Examples would be ABS, SQRT, etc.

Function Blocks are called by higher level Function Blocks or by Programs. A Program is a special case of a Function Block in that it is connected to a Task to control its’ execution. A Task is defined to run at a particular rate (50ms, 2s, etc.) or as a One-Shot when an Boolean is TRUE. EGS has not currently implemented any One Shot tasks in any standard system software. Each Program is associated with one Task. When the Task is called by the Resource Manager it in turn executes each Program associated with itself. Each program may have many function blocks embedded in it. Programs and Tasks are defined in a Resource which is the unit of code assigned to a particular iBox.

### Hierarchical Naming Conventions

Since Function Blocks can be re-used many times in a resource, there has to be some manner of referencing a particular instance of a variable. This is accomplished by referring to the particular instance of the variable using its complete hierarchical path. For example, the CD Spread for a BetaSensor is referenced as

BetaSensor1.EOS1.profstats1.ProfSig

While the CD Spread for an FSIR Sensor would be

FSIRSensor1,EOS1,profstats1.ProfSig

Programs BetaSensor and FSIRSensor both include the Function Block EOS, which includes the profstats function block, which contains the variable ProfSig. If a new sensor was created, it would include the EOS function block and so would contain a CD Spread variable as well.

It is possible for there to be multiple instances of the same function block within the same higher level block. For instance, each virtual sensor on an FSIR Sensor is handled as a separate instance of the FSIRSensor program. Each of these Programs contains an EOS function block. So the CD Spread for the first would be

FSIRSensor1.EOS1.profstats1.ProfSig

While the second would be

FSIRSensor2.EOS1.profstats1.ProfSig

The 2 goes on the second FSIR Sensor because it is the second instance of the block in the same parent block. The EOS block is a 1 in both cases because it is the first instance of EOS in the particular instance of FSIRSensor.

GCT automatically defines the instance name as the block name plus a 1 for the first instance in the parent block, a 2 for the second, etc.  However, the develop has the ability to override the automatic names. If it is cleared to call the first FSIR Sensor “Moisture” and the second “Thickness”, then there is no need for the 1 or 2. In most cases, the automatic names are kept for block names because they are generic and can be assigned on a system specific basis.

GCT will also automatically define the instance of a variable as the variable type plus the instance number, e.g. BOOL1, REAL7, INT4, etc.  In this case, the automatic name is hardly ever kept because the usage of the variable would be totally unclear. Only some spare variables are left with the automatic names.

## EGS Implementation

Standard Systems are created by creating a project. Projects are defined as a collection of function blocks made up from standard blocks along with any specialized blocks created for a specific system. Standard blocks are stored in libraries and can be used by any project. Specialized blocks are stored directly in the project for which they were created. The map file for systems built after June 2002 contain the name of the project from which they were built. This makes it easier to rebuild or debug systems when required. A log is also kept on the PC that built the binary. This log notes the time and date, as well as the configuration file and project used in the build.

### Libraries

Several libraries exist that contain reusable blocks. They are mostly separated into categories so that they can be located more easily. The sections below contain a short description of the blocks held in some of the major libraries.

#### Scanner

The Scanner library is by far the largest. It contains all the function blocks for all Frame and Sensor functions. Most Frame binaries have standard Frame blocks. A few systems have modifications for special Process I/O in the frame, but more and more configurable blocks are being created to reduce the number of system requiring special Process blocks. Each type of sensor has a corresponding Sensor Program. These programs share a great deal of standard functions, such as filtering, die mapping, Zoom functions, roll averaging, etc.  Each also has special functions that distinguish it from other – calibration equations, the actual analog signals used, etc.

#### MDCtl

The MDCtl library contains the blocks used to create standard MD Control for extrusion. These are basic blocks such as AvgCtl (used for control of Gauge average),DDCCtl (used for Line Speed and/or Screw Speed Control), Error, DeadBand and PID (which actually only does Proportional/Integral Control).

#### APC

The APC library contains all the blocks required for Automatic Profile Control. Most of these blocks are array-based since APC deals almost exclusively with arrays. For instance, the ArrayPI block is an array version of the PID block used in MD Control. This makes tuning the control for MD and CD controls very similar.

#### Calender

The calender control function blocks were developed somewhat independently and specifically for calenders, so the tuning is not based on the same equations as for extrusion.

#### EGSSNAP

This library contains the blocks used to implement the process I/O using the OPTO-22 Snap IO modules. Identical blocks are used for both the frame-based process I/O and the separate Process iBox.

#### EGSMISC

This library contains miscellaneous function blocks that don’t fit into any other category, but that are useful to the EGS applications. For example, the block that blinks the iBox LED each time the main Application Task is run resides in this library. In fact, the BlinkPrg block is a program that can be attached to whatever Task is to be used to blink the LED. This program contains the BlinkLED block that actually blinks the LED, as well as other miscellaneous utilities, such as synching the pSOS software clock with the hardware clock a few times every minute; copying any crashdump.dat file that is found in memory on startup; keeping the heartbeat variable used by Wonderware to determine communications quality, etc.

#### PROCESS

The Process Library contains the Process Function Block used in both Frame I/O and in a Process iBox. This block call the Snap I/O routines and also processes the EGS specific usage of the I/O. The Prcs program is used to call the Process b lock in a Process iBox.

#### STRUCTS

The STRUCTS library is used to define EGS defined structures used throughout the applications. This is where the data structures are defined for calibration, data passed between sensors and frames, Profile arrays, Mapped arrays, etc.

### Projects

Resources are defined in Projects. Libraries can contain all the programs and blocks, but a specific application must be completed by building a project into a loadable resource. Standard projects may often contain no blocks and use library versions for everything. Special builds for a specific customer may require copying standard blocks from a library into a project and modifying it just for that build. The projects are then archived so that specials can be recreated for a particular system.

#### Beta_EUM

The Beta_EUM project is used to build a frame with a single Beta sensor with diebolt mapping. MD Control is included but can be disabled through AIF or Setup displays.

#### Beta_Cal

Beta_Cal is also a frame with a single Beta sensor, but with calender control instead of extrusion MD Control. Since the standard Beta Sensor Program includes diebolt mapping, it is included here, but is typically not used. Zone displays are usually used instead. (The BetaSensor program includes both zone arrays and die-mapped arrays in all Beta Sensor systems).

#### IR

The IR project is a frame with a single FSIR Sensor. (i.e. a single measurement from a single sensor). It contains extrusion MD Control.

#### IR2Sen

This project is a frame with 2 measurements from a single FSIR Sensor (Basis Weight and Moisture, or top and bottom coats for example). It does not include control.

#### Calp_EUM

This project is a frame with a single Caliper sensor and extrusion MD Control.

#### CalpBeta

This project is a single frame with a Beta Sensor and a Caliper Sensor. It also includes calculations for a virtual Density sensor (BW/Caliper). It does not include control.

#### BetaIR

The BetaIR project includes a Beta Sensor and an FSIR sensor witrh a single measurement. It includes optional extrusion MD Control.

#### Ctr_Base

This project is a frame with a single Beta Sensor with no control. It is meant to be the first frame in a multi-frame Same Spot system, but it could also be used for a single Beta without control (though there is no real advantage over using the standard Beta_Eum and disabling control). The actual Same Spot code is included in the standard Frame and Beta Sensor programs and is configured through the AIF. I.E. there is nothing special required for the base frame in a Same Spot system.

#### CTR_Grss

This project is a frame with a Single Beta Sensor used to calculate a coat weight (by subtraction of a previous frame’s measurement) and Gross Weight (by using a Virtual Sensor that adds the Coat and the measurement from the previous frame). It contains optional extrusion MD Control.

#### Ctr_GCAL

This project is similar to CTR_Grss but substitutes calender control for extrusion MD Control.

#### APC_EUM

This project is used to build the standard APC iBox.

#### Prcs_EUM

This project is used to build a standard Process iBox with configurable I/O.

#### System Specific Projects

Systems that require special modifications are created as special projects and archived. They are usually named after the customer that it was created for. Often special builds are used for more than one system, and often for more than one customer.

# Application Utilities

## PC Based

### Kermit

Important commands (type the command, then press the Enter key):

INTRO    - For an introduction to MS-DOS Kermit.

VERSION  - For version and copyright information.

EXIT     - To leave MS-DOS Kermit.

Press the question-mark (?) key for context-sensitive help

at any point within a command.

[C:\KERMIT] MS-Kermit>intro

Introduction to MS-DOS Kermit

o An MS-Kermit command is a line of words separated by spaces and ending with

a carriage return <the Enter key>.  Example: SET SPEED 2400<Enter>

o Most words can be abbreviated and can be completed by pressing the Esc key.

Example: SET SPE 24<Enter>  or even  SET SPE<Esc> 24<Esc><Enter>

o Help (detailed, specific): press the "?" key where a word would appear.

o Edit lines using the Backspace key to delete characters, Control-W to delete

words, and Control-U to delete the line.  Control-C cancels the command.

o Frequently used MS-Kermit commands:

EXIT           Leave the Kermit program. QUIT does the same thing.

SET            PORT, PARITY, SPEED, TERMINAL and many other parameters.

SHOW           Display groups of important parameters. SHOW ? for categories.

CONNECT        Establish a terminal connection to a remote system or a modem.

Control-C    (Control-  followed by "C")  Return to MS-Kermit> prompt.

SEND filename  Send the file(s) to Kermit on the other computer.

RECEIVE        Receive file(s), SEND them from Kermit on the other computer.

GET filename   Ask the remote Kermit server to send the file(s) to us.

FINISH         Shut down remote Kermit but stay logged into remote system.

BYE            FINISH and logout of remote system and exit local Kermit.

o Common startup sequence: SET SPEED 9600, CONNECT, login, start remote Kermit,

put it into Server mode, escape back with Control-C, transfer files with

SEND x.txt, GET b.txt, BYE.

MS-DOS Kermit commands, a functional summary:

Local file management:         Kermit program management:

DIR    (list files)            EXIT     (from Kermit, return to DOS)

CD     (change directory)      QUIT     (same as EXIT)

DELETE (delete files)          TAKE     (execute Kermit commands from file)

RUN    (a DOS command)         CLS      (clear screen)

TYPE   (display a file)        PUSH     (enter DOS, EXIT returns to Kermit)

SPACE  (show disk space)       Ctrl-C   (interrupt a command)

Communication settings:        Terminal emulation:

SET PORT, SET SPEED            CONNECT  (begin terminal emulation)

SET PARITY                     HANGUP   (close connection)

SET FLOW-CONTROL               Alt-X    (return to MS-Kermit> prompt)

SET LOCAL-ECHO                 SET KEY  (key mapping)

SET ? to see others            SET TERMINAL TYPE, BYTESIZE, other parameters

SHOW COMMUNICATIONS, MODEM     SHOW TERMINAL, SHOW KEY

Use "?" within comands for help on what fits that word.

[C:\KERMIT] MS-Kermit>exit

For System 21, we only will use the direct serial connections to talk to an iBox. The iBox terminal serial port is set up as 9600 Baud, 1 Stop Bit, 8 Data Bits, No Parity, No Flow Control. Kermit is normally set up for 7 data Bits since the terminal emulation does not use the 8th data bit.

Typically there will be a desktop icon for Kermit. This will normally call a batch file that will start up the Kermit program for you. It will also set up the standard PC to iBox properties for you. If you need to change these defaults, you can either edit the batch file and restart or use the Kermit commands to change for one session only. Once the batch file is run, you have to connect by typing a c at the Kermit prompt.

Once connected to the iBox, the X-Shell prompt should be returned (>).  All X-Shell commands and utilities can now be used.

Since this program simulates a terminal, all iBoxdiagnostic and startup messages will be printed.

#### Commands

[C:\KERMIT] MS-Kermit>?

Ask, Askq (read keybd to variable)   Mail     (file to host Mailer)

APC text  send App Prog Cmd to host  Open Read/Write/Append file  scripts

Bye      (logout remote server)      Output text     (for scripts)

C or Connect  (become a terminal)    Pause [seconds], MPause [millisec]

Check (graphics, tcp/ip, networks)   Pop, End (exit current macro/Take file)

Clear   (Input, comms recv buffer)   Push     (go to DOS, keep Kermit)

Close    (logging and script file)   Quit     (leave Kermit)

CLS (clear screen at command level)  R or Receive  (opt local filename)

CWD or CD  (change dir &/or disk)    Read (line from a file to variable)

Decrement/Increment variable number  Reinput  (script Input, reread buffer)

Define/Assign   (a command macro)    Remote   (prefix for commands)

Delete   (a file)                    Replay   (file through term emulator)

Dial     (phone number)              Run      (a program)

Directory (filepsec)                 S, Send, Resend, Psend  local new-name

Disable  (selected server commands)  Server [timeout] (become a server)

Echo text (show line on screen)      Set      (most things)

Enable   (selected server commands)  Show     (most things)

EXIT     (leave Kermit)              Sleep time  (wait, no comms echos)

Finish   (to remote server)          Space    (free on current disk)

Get      (remote file opt new name)  Stop     (exit all Take files & macros)

Getc  (read 1 byte from kbd to var)  Take     (do a command file)

GetOK    (get Yes, OK, No response)  Telnet host port  NEW or RESUME

Goto    (label, Take file or Macro)  Test COM1 ... COM4 (check for UART)

Hangup   (drop DTR, hang up phone)   Transmit filespec [prompt] (raw upload)

If [not] <condition> <command>       Type     (a file)

I or Input [timeout] text (scripts)  Version  (show version and copyright)

INTRO  introduction to Kermit        Wait [timeout] on modem \cd \cts \dsr

Log (Packet, Session, Transaction)   Write  FILE or log file   text

### HyperTerminal

HyperTerm is a program included in most Windows NT and 2000 systems. It is a terminal emulation program that can be used in place of a terminal (or Kermit) to talk to an iBox using serial ports. The iBox terminal serial port is set up as 9600 Baud, 1 Stop Bit, 8 Data Bits, No Parity, No Flow Control. HyperTerm is usually found under Programs\Accessories\Communications in the Start Menu. It allows settings to be saved for retrieval for subsequent usage.

Once connected to the iBox, the X-Shell prompt should be returned (Supervisor>).  All X-Shell commands and utilities can now be used.

Since this program simulates a terminal, all diagnostic and startup messages will be printed.

### RexTerm

RexTerm is a terminal emulation program that works over the CMS network using Ethernet to talk to another CMS node. Typically this used to communicate to the iBox X-Shell from a PC. Since it uses CMS software to connect to an iBox, it cannot display startup messages and some diagnostic messages that are available through the serial port (using a terminal or serial terminal emulation program such as Kermit or HyperTerm).

To start a RexTerm session to an iBox, open up a DOS window on the PC. Type F: to set the default drive to F:. The type CD \resource\bin to change the default directory to resource\bin. If the IP address of the target iBox is not known, switch to the UDP Server task (using <ALT><TAB> or other method). The UDP Server will list the address for each registered node. Switch back to the DOS window and type

rexterm –node 10.1.2.3

Substituting the appropriate node address instead of 10.1.2.3. The rexterm/X-Shell prompt should be returned (the last digits of the node address plus >.  In this example 3> would be printed). All X-Shell commands and utilities can now be used. To disconnect from rexterm, type #exit. Be sure to be back at the X-Shell before typing the #exit or you may have problems connecting again later.

If you try to connect to rexterm and the connection fails, you can try typing

rexterm –node 10.1.2.3 –force

The –force parameter will close any previously disconnected sessions that are still active. If you still are not connecting, then CMS is not properly connecting to the requested node. Try another node. If that works, then the target iBox is possibly having network problems. Try rebooting that iBox. If the second node also failed, there may be CMS problems on the PC. Try unloading and loading CMS. Make sure that the Router program is running (there will not be any messages on the DOS screen for the Router in any case).

### Router

The Router (cmsrout) functions as the sender and receiver of all CMS messages on a particular node. All programs that want to send or receive data pass it through the router with an resource name and variable name and the router will figure out how to get to it. Each node in a CMS network requires a router to be running. CMS is the Communications Messaging System developed by Eurotherm Controls and used by EGS for most network communications.

### UDP Server

The UDP Server is responsible for keeping track of the status of all CMS nodes in a networked system. Any iBox or non-Server PC registers itself with the UDP Server when it starts its’ router. The UDP Server will keep the other nodes updated on the available nodes so that they can communicate with each other. For instance, when APC wants to get sensor data from a frame, it asks the UDP Server for the node address corresponding to the resource name contained in its’ variable reference (as defined in the VRF file downloaded to an iBox at startup). The UDP Server keeps known network names and addresses in a file on the PC (F:\resource\bin\nodes.cms). This file is updated periodically and is used on the restart of the UDP Server to inform the UDP Server what nodes have been previously registered. This allows all the nodes to be active without having to re-register. If the nodes.cms file is deleted and the UDP Server restarted, it will not know about any nodes until they are re-registered by restarting an iBox (or restarting the router program on a secondary PC). There should only be one UDP Server on any CMS network.

### NFP Server

The NFP Server is used to pass files between CMS nodes (using the standard Network File Protocol). When an iBox is loaded (using TFTP) it starts running the downloaded binary, then requests the NFP Server to send the AIF, VRF, RIF and RLW files. It is used whenever files are transferred between nodes.

## iBox Based

### The X-Shell

The X-Shell is a user interface supported by the Resource Manager that allows interaction with shell commands as well as an environment in which to execute the diagnostic and utility programs of the Resource Manager.

#### help

Supervisor>help

active      ,Display the status of an active task

alias       ,Set/list an alias

chmod       ,Change file permissions

config      ,Configure XSH

delete      ,Delete executable task

directory   ,File directory

diskcopy    ,Copy disk

echo        ,Echo command arguements

execute     ,Execute a task

for         ,Loop

gettime     ,Display the elapsed time

settime     ,set the system time [use GMT]

setrtc      ,set the real time clock [use GMT]

syncclocks  ,Sync s/w clock with RTC

setdate     ,Set the system date [use GMT]

graveyard   ,Display the dead tasks

help        ,Display help on a subject

if          ,Conditional execution

mkdisk      ,Make a RAM disk

list        ,List all executable tasks

powerdown   ,Simulate a powerdown

priority    ,Display/change the priority

reset       ,Reset XSH

setenv      ,Set an environment variable

setevent    ,Set an event for a task

sleep       ,Sleep for a number of seconds

stdio       ,Display ( and alter ) the standard io channels

tic         ,Display the tick rate

unalias     ,Delete an alias

unschedule  ,Unschedule a scheduled task

unset       ,Delete an environment variable

version     ,Report version number of XSH

wait        ,Wait for a task to terminate

wakeup      ,Wake up a task

while       ,Conditional loop

#### Time and Date Commands

Supervisor>gettime

Elapsed Time Since StartUp = 10 Days 05:13:36.635

System Time: 01-AUG-2002  21:00:55 GMT

Supervisor>setrtc -h 20 -m 34

RTC Date:AUG-01-2002   20:34:24 GMT

Supervisor>gettime

Elapsed Time Since StartUp = 10 Days 05:14:09.138

System Time: 01-AUG-2002  20:34:27 GMT

Supervisor>help setdate

setdate [-d <1-31>] [-m <1-12>] [-y <eg. 1999>]

#### dir

Supervisor>dir

RESOURCE.RLW

RESOURCE.VRF

RESOURCE.AIF

RESOURCE.RIF

Supervisor>dir -l -s

Directory of A:

RESOURCE.RLW     63        2002:07:23:15:41:56 rw

RESOURCE.VRF     1543      2002:07:23:15:41:57 rw

RESOURCE.AIF     16886     2002:07:23:15:42:02 rw

RESOURCE.RIF     1109      2002:07:23:15:42:04 rw

4/20 files in 41/900 blocks

#### active

Supervisor>active

Supervisor     5

loadtask       4

rourecov       3

rexserv        2

cmsrout        1

Supervisor>active -a

Supervisor     5 Supervisor

loadtask       4 loadtask task -q FastTask

rourecov       3 rourecov

rexserv        2 rexserv -i 2:2048 -timeout 20

cmsrout        1 cmsrout -u -v 5 -m UDP 10.1.0.131

#### setenv

Supervisor>setenv

CMSPROCS=cmsrout rexserv nfpserv

CMSLOAD=-i "20:148 4:512 10:1504 16:max"

CMSROUTER=-u

SYSSTATE=SYSTEM

SYSDISK=ADC:

CMSSIZE=800000

CMSNODE=10.1.0.132

RESOURCE=100F1170

CMSMEDIA=-v 5 -m UDP 10.1.0.131

CFG_CPU=10.1.0.131::f:/bootp/

RESNAME=W51586S1

#### alias

Supervisor>alias

copy=lfstool copy

del=lfstool delete

type=lfstool type

SUPERVISOR=exe -f Supervisor

CMS_LOADER=cmsload $CMSNODE -s $CMSSIZE -b 16896 $CMSLOAD

CMS_ROUTER=exe cmsrout $CMSROUTER $CMSMEDIA

CMS_LOAD=CMS_LOADER;CMS_ROUTER;NFP_LOAD;REX_LOAD

CMS_KILL=setevent 0 -all -q $CMSPROCS;wakeup -q $CMSPROCS;wait -all -q $CMSPROCS rourecov;cmsunld -q

REX_LOAD=exe rexserv -i 2:2048 -timeout 20

NFP_LOAD=

RES_STATE=resctl $RESKEY -confirm -q resource

RES_LOADER=exe -f loadtask loader -n $RESNAME -auto -q $RESCMS $RESKEY $RESLOAD $RESOURCE;wait -q loadtask

RES_TASK=exe loadtask task -q $RESKEY $RESCMS

RES_LOAD=RES_LOADER;RES_VAC;RES_RLW;RES_RIF;RES_AIF;RES_POSTLOAD;RES_SUSPEND;RES_RUN;RES_LOADED;RES_WAIT;RES_RIF;RES_AIF;RES_VRF;RES_PRERUN;RES_RESUME;RES_POSTRUN

RES_LOAD1=RES_LOADER;RES_VAC;RES_RLW;RES_RIF;RES_AIF;RES_POSTLOAD;RES_SUSPEND;RES_RUN;RES_LOADED

RES_LOAD2=RES_RIF;RES_AIF;RES_VRF;RES_PRERUN;RES_RESUME;RES_POSTRUN

RES_RUN=for t in `reslist $RESKEY -tasks` do 'RES_TASK $t'

RES_START=RES_STATE start

RES_STOP=RES_STATE stop

RES_SUSPEND=RES_STATE suspend

RES_RESUME=RES_STATE resume

RES_MODIFIED=RES_STATE modified

RES_LOADED=RES_STATE loaded

RES_TASK_KILL=if -n `active loadtask task` then "RES_STATE unload"

RES_KILL=RES_PREKILL;RES_TASK_KILL;if -n `active resproxy` then RES_KILL_PROXY;resunld $RESKEY resource -q;RES_POSTKILL

RES_RESET=RES_STATE reset;RES_RIF;RES_PRERUN;RES_STATE continue

RES_LOAD_PROXY=exe resproxy -act -q -i 1:4096

RES_KILL_PROXY=setevent 0 -q resproxy;wakeup -q resproxy;wait -q resproxy

RES_RIF=if -f ${SYSDISK}RESOURCE.RIF then "resinst -input ${SYSDISK}RESOURCE.RIF $RESKEY -q"

RES_AIF=if -f ${SYSDISK}RESOURCE.AIF then "resinst -input ${SYSDISK}RESOURCE.AIF $RESKEY -q"

RES_RLW=if -f ${SYSDISK}RESOURCE.RLW then "resinst -input ${SYSDISK}RESOURCE.RLW $RESKEY -q"

RES_VRF=if -f ${SYSDISK}RESOURCE.VRF then "resinst -input ${SYSDISK}RESOURCE.VRF $RESKEY -q"

RES_VAC=

RES_FILE_DELETE=for ext in RIF RLW VAC do 'if -f RESOURCE.$ext then "lfstool -q delete RESOURCE.${ext}"'

RES_WAIT=

AUTOEXEC=SUPERVISOR

RLW_COPY=exe gfstool -q copy ${CFG_CPU}${RESNAME}.rlw A:RESOURCE.RLW;wait -q gfstool

VRF_COPY=exe gfstool -q copy ${CFG_CPU}${RESNAME}.vrf A:RESOURCE.VRF;wait -q gfstool

AIF_COPY=exe gfstool -q copy ${CFG_CPU}${RESNAME}.aif A:RESOURCE.AIF;wait -q gfstool

RIF_COPY=exe gfstool -q copy ${CFG_CPU}${RESNAME}.rif A:RESOURCE.RIF;wait -q gfstool

SYS_CONFIG=setenv SYSSTATE=CONFIG

SYS_SYSTEM=setenv SYSSTATE=SYSTEM

SYS_MONITOR=RES_KILL;CMS_KILL;reset -monitor

SYS_RESET=RES_KILL;CMS_KILL;reset -init

SYS_SHUTDOWN=SYS_BACKUP;SYS_MONITOR

RES_DOWNLOAD=RES_KILL;RES_DELETE;download -export RESOURCE

RES_DELETE=unset RESOURCE

RES_HALT=RES_SUSPEND;resctl resume task DefaultTask -q;resctl start task DefaultTask -q

RES_POSTLOAD=

RES_POSTRUN=

RES_PRERUN=

### resinst

resinst is the resource instance data tool. It is used to read or write variables in the resource. It can be used with typed commands at the keyboard or with a file containing change commands (like the AIF, RIF, VRF and RLW files loaded at startup).

The resinst commands can be typed from the X-Shell (whether using a terminal, a terminal emulation program or rexterm). The only difference is that commands containing the character # need to be modified slightly when using rexterm., Since the # character is a special break character to rexterm, # characters must be replaced with ## when using rexterm.

To use resinst with a file, type the command

resinst –input resource.aif

where resource.aif is a file containing variable change commands.

resinst can also be used for a single command, as in the example

resinst –command Frame1.Scan11.ScanPos1.NoRef:=TRUE

This will execute a single command and return to he X-Shell.

If multiple commands are going to be executed, the edit mode of resinst can be used by typing

resinst –edit

The edit mode allows setting a default path to a function block. Subsequent commands can be relative to the default. A period (.) typed at the end of a function block leaves the default at that level. A period at the beginning of the next command starts at the default. The Greater than symbol (>) is the prompt character. See the sample session below (Typed commands are in Bold, responses are in italics):

resinst –edit

>Frame1.

Frame1>.Scan11.

Frame1.Scan11>.ScanPos1.

Frame1.Scan11.ScanPos1>.NoRef

Frame1.Scan11.ScanPos1.NoRef : BOOL = TRUE ;

Frame1.Scan11.ScanPos1>

Typing a left bracket ({) causes the default to go up one level.

Frame1.Scan11.ScanPos1>}

Frame1.Scan11>

Typing a command without a period will ignore the current default. If the command does not end in a period, the default is not changed. If it does end in a period, the default path is changed.

Frame1.Scan11>BetaSensor1.NucData1.NumRdgs

BetaSensor1.NucData1.NumRdgs : DINT = 257 ;

Frame1.Scan11>BlinkPrg1.

BlinkPrg1>

As you can see, hitting a return after a variable will display the current value of the variable. To change the value, use  :=  to indicate a change

BlinkPrg1>.NeedSetup := TRUE

BlinkPrg1.NeedSetup : BOOL = FALSE ;

BlinkPrg1.NeedSetup : BOOL = TRUE ;

BlinkPrg1>

The response will show the value before the command is executed and after.

A question mark character (?) will cause resinst to list all the variables in the current function block. Two questions marks (??) will list all the variables but will truncate the array listings to the first 50 values. This is useful in some cases since arrays can have up to 9000 values, and there can be several arrays in a function block.

Output variables cannot be modified, only displayed.

To exit resinst, type the exit command. This will return the session to the X-Shell.

BlinkPrg1>exit

>

See below for a sample resinst session (Bold characters are user input).

Supervisor>resinst -edit

Warning - TASK "FastTask" still running

The Resource Instance Data Editor, Version 3.1a

(c) Copyright 1992-1998 Eurotherm Gauging Systems, Inc.

>?

[1.0.0] INTERNAL FastTask : TASK EUM ;

[2.0.0] INTERNAL Frame1 : PROGRAM Frame ;

[3.0.0] INTERNAL Globals1 : PROGRAM Globals ;

[4.0.0] INTERNAL BlinkPrg1 : PROGRAM BlinkPrg ;

[5.0.0] INTERNAL BetaSensor1 : PROGRAM BetaSensor ;

[6.0.0] INTERNAL CIOProg1 : PROGRAM CIOProg ;

[7.0.0] INTERNAL CalCont1 : PROGRAM CalCont ;

>BetaSensor1.

BetaSensor1>Frame1.

Frame1>.Scan11.

Frame1.Scan11>.ScanPos1.

Frame1.Scan11.ScanPos1>.NoRef

[2.68.34] INTERNAL Frame1.Scan11.ScanPos1.NoRef : BOOL = FALSE ;

Frame1.Scan11.ScanPos1>}

Frame1.Scan11>BetaSensor1.NucData1.NumRdgs

[5.18.31] OUTPUT BetaSensor1.NucData1.NumRdgs : DINT = 257 ;

Frame1.Scan11>BlinkPrg1.

BlinkPrg1>.NeedSetup:=TRUE

[4.0.13] INTERNAL BlinkPrg1.NeedSetup : BOOL = FALSE ;

[4.0.13] INTERNAL BlinkPrg1.NeedSetup : BOOL = TRUE ;

BlinkPrg1>exit

### resvrf

Supervisor>resvrf

usage : [resource|task <TaskName>] [-brief|-full] [-diag] [-list [-dynamic]]

[-oot] [-pst] [-y <Key>]

resvrf: [1]

Supervisor>resvrf resource -brief

Resource "W51586S1"

VAR REFERENCE Diagnostics for "FastTask"

ReqMatch             =  1055091

ReqRead              = 417112477

ResMatch             =      144

ResRead              = 417112477

MatchError           =       54

MatchTimeout         =  1054869

BadStatus            =       54

ScanOverRun          = 296433351

Supervisor>resvrf resource –list

(Note: This sample printout is truncated to the first 5 references for space reasons)

Resource "W51586S1"

VAR REFERENCE Listing for "FastTask"

[2.0.43] Frame1.Rate

^ref             = 'FastTask.ACT_INTERVAL'

^resolution      = 1 (Ultimate)

^scan            = T#100ms

^timeStamp       = DT#2002-08-02-20:40:39

^qtimeStamp      = QT#0ms

^status          = 0 (Ok)

^readStatus      = 0 (Ok)

^writeStatus     = 3 (Undefined)

^dontWrite       = 0

^propertyProtect = 1

^newRead         = 1

[2.4.25] Frame1.FrameOutput1.SpliceTestRef

^ref             = 'Frame1.Process1.SpliceTest'

^resolution      = 1 (Ultimate)

^scan            = T#50ms

^timeStamp       = DT#2002-08-02-20:40:40

^qtimeStamp      = QT#0ms

^status          = 0 (Ok)

^readStatus      = 0 (Ok)

writeStatus     = 3 (Undefined)

^dontWrite       = 1

^propertyProtect = 1

^newRead         = 1

[2.4.29] Frame1.FrameOutput1.OffShtCmdref

^ref             = 'Frame1.ScanMode1.OffShtCmd'

^resolution      = 1 (Ultimate)

^scan            = T#50ms

^timeStamp       = DT#2002-08-02-20:40:41

^qtimeStamp      = QT#0ms

^status          = 0 (Ok)

^readStatus      = 0 (Ok)

^writeStatus     = 3 (Undefined)

^dontWrite       = 1

^propertyProtect = 1

^newRead         = 1

[2.4.40] Frame1.FrameOutput1.OffShtIn

^ref             = 'Frame1.ScanMode1.OffShtIn'

^resolution      = 1 (Ultimate)

^scan            = T#50ms

^timeStamp       = DT#2002-08-02-20:40:42

^qtimeStamp      = QT#0ms

^status          = 0 (Ok)

^readStatus      = 0 (Ok)

^writeStatus     = 3 (Undefined)

^dontWrite       = 1

^propertyProtect = 1

^newRead         = 1

[2.4.45] Frame1.FrameOutput1.TimeToStdz

^ref             = 'Frame1.Scan11.AStandard'

^resolution      = 1 (Ultimate)

^scan            = T#50ms

^timeStamp       = DT#2002-08-02-20:40:43

^qtimeStamp      = QT#0ms

^status          = 0 (Ok)

^readStatus      = 0 (Ok)

^writeStatus     = 3 (Undefined)

^dontWrite       = 0

^propertyProtect = 1

^newRead         = 1

### gfstool

Supervisor>gfstool

Task "gfstool" must be executed with "exe"

Supervisor>exe gfstool

usage: copy|delete|type|dump|concat|compare <src> [<dst>]

[-binary] [-i <InitData>] [-y <Key>] [-quiet]   (from gfstool)

Supervisor>exe gfstool copy resource.rif ${CFG_CPU}temp.tmp

The GFS Tool,  Version 11.0f

(c) Copyright 1992-1998 Eurotherm Gauging Systems, Inc.

For convenience, there are several alias commands available for gfstool.

Supervisor>type resource.vrf

The LFS Tool, Version 1.0f

(c) Copyright 1992-1998 Eurotherm Gauging Systems, Inc.

MDControl1.GetPVs1.AverageRef^ref := 'BetaSensor1.EOS1.HiResAvg' ;

MDControl1.GetPVs1.AverageRef^scan := T#25ms ;

MDControl1.GetPVs1.LineSpeedRef^ref := 'Frame1.Process1.LineSpeed' ;

MDControl1.GetPVs1.LineSpeedRef^scan := T#25ms ;

MDControl1.GetPVs1.ScrewSpdRef^ref := 'Frame1.Process1.ScrewSpeed' ;

MDControl1.GetPVs1.ScrewSpdRef^scan := T#25ms ;

MDControl1.GetPVs1.NumScansRef^ref := 'BetaSensor1.NucData1.NumScans' ;

MDControl1.GetPVs1.NumScansRef^scan := T#25ms ;

MDControl1.GetPVs1.DeltaTimeRef^ref := 'FastTask.ACT_INTERVAL' ;

MDControl1.GetPVs1.DeltaTimeRef^scan := T#25ms ;

Frame1.Process1.GetPOs1.TimOut0Ref := 'MDControl1.LSControl.outNPls' ;

Frame1.Process1.GetPOs1.TimOut1Ref := 'MDControl1.LSControl.outPPls' ;

Frame1.Process1.GetPOs1.TimOut2Ref := 'MDControl1.SSControl.outNPls' ;

Frame1.Process1.GetPOs1.TimOut3Ref := 'MDControl1.SSControl.outPPls' ;

# Use the following for Direct Output from thickness #

# Frame1.Process1.GetPOs1.TimOut0Ref := 'MDControl1.DirectOut.outNPls' ;

# Frame1.Process1.GetPOs1.TimOut1Ref := 'MDControl1.DirectOut.outPPls' ;

#

# Biax Specific references:

#

MDControl1.GetPVs1.CascScansRef^ref       := 'W51586S2:BetaSensor1.EOS1.NumScans' ;

MDControl1.GetPVs1.AvgDeltaRef^ref        := 'W51586S2:MDControl1.AvgCtl1.DDCDelta' ;

BetaSensor1.EOS1.ProfTrim1.LoTrimCalc^ref := 'W51586S2:BetaSensor1.MassMap1.CastLoTrim' ;

BetaSensor1.EOS1.ProfTrim1.HiTrimCalc^ref := 'W51586S2:BetaSensor1.MassMap1.CastHiTrim' ;

Frame1.VRFLoaded := TRUE ;  # Must be last line of VRF file #

Supervisor>dir

RESOURCE.RLW

RESOURCE.VRF

RESOURCE.AIF

RESOURCE.RIF

Supervisor>copy resource.rlw temp.tmp

The LFS Tool, Version 1.0f

(c) Copyright 1992-1998 Eurotherm Gauging Systems, Inc.

Supervisor>dir -l -s

Directory of A:

RESOURCE.RLW     63        2002:07:23:15:41:56 rw

RESOURCE.VRF     1543      2002:07:23:15:41:57 rw

RESOURCE.AIF     16886     2002:07:23:15:42:02 rw

RESOURCE.RIF     1109      2002:07:23:15:42:04 rw

TEMP.TMP         63        2002:08:02:20:39:32 rw

5/20 files in 42/900 blocks

Supervisor>del temp.tmp

The LFS Tool, Version 1.0f

(c) Copyright 1992-1998 Eurotherm Gauging Systems, Inc.

"temp.tmp" deleted

Supervisor>dir

RESOURCE.RLW

RESOURCE.VRF

RESOURCE.AIF

RESOURCE.RIF