Memorandum

| To: | Distribution |
| --- | --- |
| From: | Jim Twyman |
| Date: | April 21, 2006 |
| Subject: | egsTransfer Utility |

With the July 2006 release of System 21Plus! Software, a new file transfer utility will be available. The utility will aid EGS Gauging personnel in updating the iBox or dBox Linux Executable (LEX) application program and other files that currently resides on the USB Mass Storage Drive (Jump Drive) that is connected to each iBox/dBox. Previously, in the System 21 product, the binaries were held in the F:\bootp\ folder and downloaded upon power up of the EUM. Updating a EUM binary was a matter of copying the new binary into the F:\bootp\ folder and then accessing the EUM reset or frame power switch to force a reload. This new utility will allow updating and control from over the network.

The program is a modified File Transfer Protocol (FTP) with a client application on the PC and a FTP server application running in the background on the iBox/dBox.

Figure 1 - Starting egsTransfer

The first action is to connect to the iBox. If this is the first time, then click in the IP Address box and enter in the IP Address. Otherwise, previous entries are saved and just go select it from the list. Below this window is the Initial Directory to access on the iBox. Normally, this is the USB Jump Drive which has the pathname of “/mnt/jump/”. If this is not shown, click the ‘Reset’ button to the right. Next, click the ‘Connect’ button to have a network connection to the iBox made. With the connection established with the iBox, egsTransfer will request the directory listed in the Initial Directory box.

Figure 2 - After Connection Established

The following is a list of what the functions may be performed by egsTransfer.

# Connect

As described above, this button will make the initial connection to the iBox.

# Disconnect

When all activities have been completed with this iBox and the user wants to connect to a different iBox, use this button. Once this button is used, another iBox IP Address may be entered or selected from the IP Address box. If use of egsTransfer is complete, the exit ‘X’ in the upper right corner of the window also performs a Disconnect prior to termination.

# Directory Window

When a connection is established with the iBox, the current directory is shown in this display. The Name, File Size in bytes and Last Accessed Date is shown for files. A directory will only have a Name and a label (“< Directory >”) to show what it is. Clicking on the directory icon or Name will move to that directory. The “..” directory indicates to go UP one level. Clicking all other Named directories will move DOWN into the sub-directory. At all times, the current directory is shown just above the Directory Window.

# Get File

This button will get the selected file from the iBox and copy it to a selected destination on the PC.

# Send File

This button will copy the selected file from the PC to the iBox and place it in the current iBox directory.

# Delete File

This button will delete the selected file from the iBox. A confirmation box will be presented to allow the user to verify this action.

# Exec File

Selecting the file and then clicking this button will execute programs or script files. This is obviously a potentially dangerous action and not to be used unless directed. The program or script file shown executes and exits by itself.

# Kill LEX

This button allows the user to terminate the application file running on the iBox. The iBox’s Watch Dog Reset Timer functionality is also disabled to prevent immediate rebooting of the iBox. This action must be performed before sending a new LEX file to the iBox.

# Reboot

After a new LEX file has been sent to the iBox, this action will reboot the iBox

# Refresh Directory

Requests a new directory from the iBox.

# Backup iBox

This button will begin the actions to copy the current directory in the iBox up to a specified folder on the PC.

- Upon clicking the button, the user is prompted for the folder to place the backup in. (Note: Sometimes, when creating New Folders, verify the name you give it is shown in this and the next dialogs.)

Figure 3 – Backup folder

- A Confirmation is presented to review the from and to locations. This dialog shown should be checked carefully.

Figure 4 – Backup Transfer Confirmation Dialog

- If there were files already in the backup folder, the user is prompted to Cancel or have all files deleted.

Figure 5 – Delete old backup files Confirmation Dialog

- The transfer is usually completed in a one or two seconds.  Once complete, a Status line displays how many files were copied.

Figure 7 –Backup Completed

# Restore iBox

This button will begin the actions to copy the specified backup directory to the current directory in the iBox. This is another potentially dangerous action to perform.

- First, if this is the ‘/mnt/jump/’ iBox folder and the system is running, then it should be taken off sheet via the either the Wonderware screen or the Offsheet switch.

- Use the ‘Kill LEX’ button to terminate the current LEX application and disable the Watch Dog Reset Timer routines.

- Click the ‘Restore iBox’ button. This performs some checks and displays a general Dialog as shown below.

Figure 8 –Restore Setup and Confirmation

- There are a number of check boxes. The first is just a confirmation on what you are going to perform.

- The second is shown because the current directory is not the typical location ‘/mnt/jump/’. This check is not shown normally.

- The recommended action is to delete all the files in the iBox directory prior to sending the backup down.

- The last check is a confirmation that a special data file kept with the backup on the PC was not found. This data file contains the exact (case sensitive) filenames when the original backup was made. This is not critical, but may be more important in the future.

- Browse – This button brings up a dialog to allow selection of the PC Backup folder to be used as the source directory.

(Note: All check boxes must be checked prior to the OK button being enabled for use.)

- The Confirmation Dialog presents the final chance to abort the operation. The user must verify the Source (PC) directory and the Destination (iBox) directory carefully before clicking the OK button.

Figure 9 –Final Confirmation Dialog

- Once the backup has completed, the user may click the ‘Reboot’ button to reboot the iBox.

# Notes:

  - The program does not:

    - Delete directories.

    - Copy a directory by selecting a directory and then ‘Get File’, ‘Send File’, …

    - Execute a directory.

    - Recursive directory anything.

    - Restore deleted files on iBox. Once there are deleted, they are truly gone.