#### Memorandum

| To: | Distribution |
| --- | --- |
| From: | Jim Twyman |
| Date: | January 21, 2006 |
| Subject: | System 21Plus! Crash Dump |
| Cc: |  |

### Introduction

Why is a crash dump required? A crash dump is required in the extremely very rare case that there is a software problem or more likely a hardware problem that results in Linux abruptly terminating the EGS application. To trace the fault, Linux provides the core dump facility. This in practice creates a file with the image of memory at the time of the fault. This file could be loaded into gdb (GNU Debugger) for examination of the software state. This binary file has been as large as 48 Mbytes in size. Therefore, another kernel change was taken on to reduce the file to just a small text file with essential information contained within. This document announces the current version of the crash dump to be incorporated into the Linux kernel used by System21Plus.

### Enabling Crash Dump

Linux provides a command that control the maximum size of a core dump (crash) file that may be written to disk media. This is the ulimit command that we originally used during early debugging of our application port. The Linux kernel was modified to check this limit and should it be set to a particular size, then the EGS crash dump file would be generated rather that a standard Linux core dump. To enable the EGS crash dump, the following command must be executed in the ‘startm22’ startup script.

ulimit  -c  999

### Crash Dump Test

A special program was written to test the crash dump by doing an optional divide-by-zero and write using a null pointer. (Should our scanner or APC Linux EXecutable (lex) ever crash, I will test on that too.) The program is executed as follows:

$ ./crasher -F

Compiled: Dec 20 2005 at 15:47:38

Copyright (c) 2005 EGS Gauging Inc.

version 2.0.0 (TimeSys Linux-based)

Flts: 1.000000, 3.641593

Segmentation fault (core dumped)

### The Crash Dump File

When a segmentation fault occurs (along with other fatal Linux signals), a core dump may be generated. Again, this is controlled by the ulimit command that has been set to 999. As a side note, the size maximum is calculated to be 999 x 512 bytes, but the number is used as a flag so the file will not actually be that large. Its size is actually nearer to 2600 bytes. During the writing of the crash dump file, some limitations of being in the kernel prevented creation of a fully human readable file. Thus, the crash dump file has some data that needs further processing at some later point outside the kernel before it is fully human readable. Initially, the crash dump is written into the (RAM disk) root file system /usr/ folder. Another program is required to safely save it on the iBox/dBox USB Mass Storage Drive. A separate file transfer program is available to copy the file to the PC over the network. The data presented in Figure 1 shows the raw crash dump file.

----------------------------------------------------------------------------

Crash Dump:                             Time  R:  1136304520

File   : crasher

Pid    :      157 ,   Parent Pid :      151

Signal : SIGSEGV(11) - Segmentation violation

Registers: .................................................................

Address: 0x080A525C

0)     ebx = 4021F9A4 :  1075968420    1)     ecx = 00000000 :           0

2)     edx = 00000000 :           0    3)     esi = 40014B6C :  1073826668

4)     edi = BFFFFE94 :  3221225108    5)     ebp = BDFFF884 :  3187669124

6)     eax = 12345678 :   305419896    7)     xds = 0000002B :          43

8)     xes = 0000002B :          43    9)     xds = 0000002B :          43

10)     xds = 0000002B :          43   11) ori_eax = FFFFFFFF :  4294967295

12)     eip = 08048ABB :   134515387   13)     xcs = 00000023 :          35

14)   eflag = 00010202 :       66050   15)     esp = BDFFF83C :  3187669052

16)     xss = 0000002B :          43

Stack:(esp)

40a0da48 00000003 bdfff874 080e924e 40a0da48 00000000 620396c0 40411186

000007d7 00000026 037f0c7f 00000000 0002be99 00000000 00096700 0000010a

40699cc8 00000000 bdfff8c4 0808212e 406c56fc bdfff8dc bdfff8b4 0002be99

04a0da48 40642350 bdfff8d4 080ee1b9 40a0da48 00000000 bdfff8c0 00000001

4064676c 00000000 bdfff8f4 08067308 40699cc8 40996afc bdfff914 080d9f29

40a0da48 bdfff8f0 0002c04d 0102c04d 406eb890 40642100 bdfff914 080dc33a

4064676c 0102be99 40642358 40642500 00000001 406eb890 bdfff934 080df089

406eb890 00000000 00000000 00036244 40642500 40642344 bdfff954 080d1909

FPU Registers: .............................................................

fctrl = 037F       fstat = 0000        ftag = 8AA8       fioff = 0023

fiseg = A148       fooff = 002B       foseg = 1F80       mxcsr = 1F80

ST0  = 00000000000000000000

ST1  = 00000000000000000000

ST2  = 00000000000000000000

ST3  = 00000000000000000000

ST4  = 00000000000000000000

ST5  = 00000000000000000000

ST6  = 4015800BF60000000000

ST7  = 7FFF8000000000000000

FPU Register Dump:(14 shorts)

037F 0004 0000 011D 8AA8 0804 0023 0000 A148 0804 002B 0000 1F80 0000

Instruction :  0xEC830289

Sequence    :  0x0289CC45 0x830289CC 0xEC830289 0x0CEC8302 0x680CEC83

Figure 1. Raw Crash Dump

### Crash Dump Processing

The raw crash dump is complete and readable except for two sections. In the kernel, the standard glibc routines maybe could be used, but the possibility of causing a second fault that would catapult the Linux OS into a fatal Oops state, was to be avoided. Therefore, it was decided to leave some crash dump processing to later.

The first raw data value is the time of the crash. This was left as the ‘Epoch’ time value. This was a simple conversion call that could be performed later. The second data value is something that was not available on System21, the floating-point stack and flags. In this case, the stack is written as eight 80-bit hex numbers. In the processing, the 80-bit numbers are converted to 64-bit doubles and added to the line along with some tests on the data.

The crash dump is processed using a new utility. Currently, it looks for a file named /usr/core.pid and writes the processed copy to /mnt/jump/crashdumps.txt, appending it to the end of the file. Linux uses the process ID number (pid) as the file extension.

### Final Crash Dump File

The final crash dump file is presented below in Figure 2.

----------------------------------------------------------------------------

Crash Dump:                             Time   : Tue Jan  3 16:08:40 2006

File   : crasher

Pid    :      157 ,   Parent Pid :      151

Signal : SIGSEGV(11) - Segmentation violation

Registers: .................................................................

Address: 0x080A525C

0)     ebx = 4021F9A4 :  1075968420    1)     ecx = 00000000 :           0

2)     edx = 00000000 :           0    3)     esi = 40014B6C :  1073826668

4)     edi = BFFFFE94 :  3221225108    5)     ebp = BDFFF884 :  3187669124

6)     eax = 12345678 :   305419896    7)     xds = 0000002B :          43

8)     xes = 0000002B :          43    9)     xds = 0000002B :          43

10)     xds = 0000002B :          43   11) ori_eax = FFFFFFFF :  4294967295

12)     eip = 08048ABB :   134515387   13)     xcs = 00000023 :          35

14)   eflag = 00010202 :       66050   15)     esp = BDFFF83C :  3187669052

16)     xss = 0000002B :          43

Stack:(esp)

40a0da48 00000003 bdfff874 080e924e 40a0da48 00000000 620396c0 40411186

000007d7 00000026 037f0c7f 00000000 0002be99 00000000 00096700 0000010a

40699cc8 00000000 bdfff8c4 0808212e 406c56fc bdfff8dc bdfff8b4 0002be99

04a0da48 40642350 bdfff8d4 080ee1b9 40a0da48 00000000 bdfff8c0 00000001

4064676c 00000000 bdfff8f4 08067308 40699cc8 40996afc bdfff914 080d9f29

40a0da48 bdfff8f0 0002c04d 0102c04d 406eb890 40642100 bdfff914 080dc33a

4064676c 0102be99 40642358 40642500 00000001 406eb890 bdfff934 080df089

406eb890 00000000 00000000 00036244 40642500 40642344 bdfff954 080d1909

FPU Registers: .............................................................

fctrl = 037F       fstat = 0000        ftag = 8AA8       fioff = 0023

fiseg = A148       fooff = 002B       foseg = 1F80       mxcsr = 1F80

ST0  = 00000000000000000000      =>                    0

ST1  = 00000000000000000000      =>                    0

ST2  = 00000000000000000000      =>                    0

ST3  = 00000000000000000000      =>                    0

ST4  = 00000000000000000000      =>                    0

ST5  = 00000000000000000000      =>                    0

ST6  = 4015800BF60000000000      =>     4195835.00000000

ST7  = 7FFF8000000000000000      ==========>    INFINITY

FPU Register Dump:(14 shorts)

037F 0004 0000 011D 8AA8 0804 0023 0000 A148 0804 002B 0000 1F80 0000

Instruction :  0xEC830289

Sequence    :  0x0289CC45 0x830289CC 0xEC830289 0x0CEC8302 0x680CEC83

Figure 2. Processed Crash Dump

### Parts of a Crash Dump

The crash dump file shows the state of the software at the point where a fault signal was generated. The sample presented in Figure 3 is expanded on below.

1

----------------------------------------------------------------------------

2

Crash Dump:                             Time   : Tue Jan  3 16:08:40 2006

3

File   : crasher

4

Pid    :      157 ,   Parent Pid :      151

Signal : SIGSEGV(11) - Segmentation violation

Registers: .................................................................

5

Address: 0x080A525C

0)     ebx = 4021F9A4 :  1075968420    1)     ecx = 00000000 :           0

2)     edx = 00000000 :           0    3)     esi = 40014B6C :  1073826668

4)     edi = BFFFFE94 :  3221225108    5)     ebp = BDFFF884 :  3187669124

6)     eax = 12345678 :   305419896    7)     xds = 0000002B :          43

8)     xes = 0000002B :          43    9)     xds = 0000002B :          43

10)     xds = 0000002B :          43   11) ori_eax = FFFFFFFF :  4294967295

12)     eip = 08048ABB :   134515387   13)     xcs = 00000023 :          35

14)   eflag = 00010202 :       66050   15)     esp = BDFFF83C :  3187669052

16)     xss = 0000002B :          43

6

Stack:(esp)

40a0da48 00000003 bdfff874 080e924e 40a0da48 00000000 620396c0 40411186

000007d7 00000026 037f0c7f 00000000 0002be99 00000000 00096700 0000010a

40699cc8 00000000 bdfff8c4 0808212e 406c56fc bdfff8dc bdfff8b4 0002be99

04a0da48 40642350 bdfff8d4 080ee1b9 40a0da48 00000000 bdfff8c0 00000001

4064676c 00000000 bdfff8f4 08067308 40699cc8 40996afc bdfff914 080d9f29

40a0da48 bdfff8f0 0002c04d 0102c04d 406eb890 40642100 bdfff914 080dc33a

4064676c 0102be99 40642358 40642500 00000001 406eb890 bdfff934 080df089

406eb890 00000000 00000000 00036244 40642500 40642344 bdfff954 080d1909

7

FPU Registers: .............................................................

fctrl = 037F       fstat = 0000        ftag = 8AA8       fioff = 0023

fiseg = A148       fooff = 002B       foseg = 1F80       mxcsr = 1F80

ST0  = 00000000000000000000      =>                    0

ST1  = 00000000000000000000      =>                    0

ST2  = 00000000000000000000      =>                    0

ST3  = 00000000000000000000      =>                    0

ST4  = 00000000000000000000      =>                    0

8

ST5  = 00000000000000000000      =>                    0

ST6  = 4015800BF60000000000      =>     4195835.00000000

ST7  = 7FFF8000000000000000      ==========>    INFINITY

9

FPU Register Dump:(14 shorts)

037F 0004 0000 011D 8AA8 0804 0023 0000 A148 0804 002B 0000 1F80 0000

10

Instruction :  0xEC830289

Sequence    :  0x0289CC45 0x830289CC 0xEC830289 0x0CEC8302 0x680CEC83

Figure 3. Processed Crash Dump

- The time of the crash. (UTC)

- The program filename that caused the crash.

- The Process ID of the thread along with the parent process that originally created this thread.

- The type of signal fault that forced this crash. In this case, it was illegal memory access.

- The x86 registers at the time of the crash. From this information, the eip register tells us what the instruction address is and this is listed first as it is the most helpful number in locating the fault in the software. For each register, the contents are shown in both hex and decimal unsigned long numbers.

- The top 64 items on the thread’s stack at the time.

- The FPU control registers that may or may not be accurate.

- The x87 floating point stack copied from the processor. After processing, the 80-bit (10-byte) hex numbers are converted to 64-bit doubles and printed. Some glibc functions are employed - isnan() & isinf(), to detect NANs and Infinity flag states for each number during conversion. In this test, the crasher program did a divide by zero, but that did not cause a signal fault. The signal occurred shortly after when the write using a null pointer was performed.

- The FPU registers are dumped as an array of hex numbers until it is known if there were interpreted correctly up in line 7 above.

- The x86 instruction at the address of the fault is presented. This is followed by a sequence of numbers showing two instructions (or data) values before and after this instruction address.

### What may a Crash Dump provide for information:

The crash dump along with the application’s ‘.map’ file should be sent to the R&D software group at EGS Gauging in Wilmington, Massachusetts. The files are read into a crash dump analyzer program that will assist in locating the point in the software where the crash occurred. The R&D software group should be able to either directly pinpoint the reason or at least be able to run tests to reproduce the problem.