Purpose

This Technical Note discusses a feature available beginning in compact flash made June 2006 and afterwards. This Technical Note applies to System 21Plus! Systems only.

Description

The compact flash can make and hold a backup copy of the contents of the USB Mass storage drive. This may be used for the following reasons.

- Hold a backup of the USB Mass storage drive. This is done once initially, and then only upon execution of a command script.

- Should the USB Mass storage drive fail or be missing, the iBox/dBox will use the backup copy and execute the application lex contained in this backup folder.

Therefore, it is important to have a valid and recent backup on the Compact Flash. A command script exist that may be run to refresh the Compact Flash backup. The script may be accessed in either of two ways.

- The standard telnet network terminal from most PCs:  telnet   <IP Address>

- The egsTransfer file transfer utility program on EGS PCs: egsTransfer.exe

Procedure using telnet:

1) Open a DOS Window and enter the command:   telnet  <IP_Address>

telnet 10.2.23.43

BusyBox v1.01 (2006.01.12-20:26+0000) Built-in shell (ash)

Enter 'help' for a list of built-in commands.

2) Enter command script:   /usr/local/bin/backup_USB_to_CF

# /usr/local/bin/backup_USB_to_CF

--------------------------------------------------------------

-------- Backup_USB_to_CF ----- { Thursday, May 04, 2006 } ---

--------------------------------------------------------------

Begin USB Jump Drive Check:

Check for already mounted USB Jump Drive...

/dev/jump on /mnt/jump type vfat (rw)

USB Stick is already Mounted

...Checking for /mnt/jump/myip.txt

...Checking for /mnt/jump/apcdiagnose

No APC file found. Assume this is a Scanner USB drive

...Checking for Scanner File : /mnt/jump/startm22

...Checking for Scanner File : /mnt/jump/m22conf

...Checking for Scanner File : /mnt/jump/egs.o

...Checking for Scanner File : /mnt/jump/stratusX.o

...Checking for Scanner File : /mnt/jump/tracer.o

...Checking for Scanner File : /mnt/jump/s54321s1.lex

...Checking for Scanner File : /mnt/jump/s54321s1.map

USB Jump Drive and files check : Successful

Begin Compact Flash Check:

First, Trying : /dev/ide/host0/bus1/target0...

Mount Compact Flash : Successful !

Folder : /mnt/flash/backup/ was found.

...Checking for /mnt/flash/backup/MyIP.txt

...Checking for /mnt/flash/backup/apcdiagnose

No APC file found. Assume this is a Scanner USB drive

...Checking for Scanner File : /mnt/flash/backup/startm22

...Checking for Scanner File : /mnt/flash/backup/m22conf

...Checking for Scanner File : /mnt/flash/backup/egs.o

...Checking for Scanner File : /mnt/flash/backup/stratusX.o

...Checking for Scanner File : /mnt/flash/backup/tracer.o

...Checking for Scanner File : /mnt/flash/backup/s54321s1.lex

...Checking for Scanner File : /mnt/flash/backup/s54321s1.map

Compact Flash and files check : Successful

Now Deleting old backup files and folders on Compact Flash...

Copying files from /mnt/jump/ to /mnt/flash/backup/...

Completed.

1 -rwxr-xr-x    1 0        0              45 Feb  9 18:22 MyIP.txt

7 -rwxr-xr-x    1 0        0            6370 Jun 13 03:26 crashdumps.txt

17 -rwxr-xr-x    1 0        0           15807 Jun 22  2005 egs.o

1 -rwxr-xr-x    1 0        0             801 Jan 25 08:43 m22conf

1 -rwxr-xr-x    1 0        0             689 Jun 22  2005 m22conf.bas

1 -rwxr-xr-x    1 0        0              83 Jun 22  2005 m22conf.hdr

1 -rwxr-xr-x    1 0        0             234 Jan 16 20:56 profile

1 -rwxr-xr-x    1 0        0             479 Jun 22  2005 rcb

1736 -rwxr-xr-x    1 0        0         1768531 Jun  7 15:05 s54321s1.lex

738 -rwxr-xr-x    1 0        0          751470 Jun  6 18:40 s54321s1.map

1 -rwxr-xr-x    1 0        0             826 Apr 18 09:03 startm22

48 -rwxr-xr-x    1 0        0           47898 Jun 22  2005 stratusX.o

33 -rwxr-xr-x    1 0        0           32068 Jun 22  2005 tracer.o

Dismounting Compact Flash ...

Dismounting : Successful !

Done.

--------------------------------------------------------------

---- Backup_USB_to_CF script completed -----------------------

--------------------------------------------------------------

#

3) Enter command to exit the telnet connection:  exit

#exit

Procedure using egsTransfer :

- Start the egsTransfer program and enter the IP Address in the text box. Click the ‘Connect’ button to the right and wait for the connection to be established. The default directory shown is the /mnt/jump which happens to be the USB Mass Storage drive.

- In the directory , the top folder shown should be labeled ‘..’ which indicates this is a path to the next top-level directory. Double click this icon and wait for the new directory to be displayed. This current directory is now /mnt. Now double click the same ‘..’ icon again to finally come up to the top level directory ‘/’.

- Now in the same directory window, scroll down nearly to the bottom and find the usr/ folder icon. Double click this icon to enter that folder. Next double click on the local/ and then the bin/ folder to end up in the /usr/local/bin folder.

- The script file  backup_USB_to_CF  should be directly visible in the directory window. Click once on this file to select it. Then click the ‘EXEC’ button and confirm the action in the following dialog. The backup should be complete in ten seconds.

- The program may now be exited.