# Introduction

The SNAP I/O software is responsible for low speed analog and digital input and output processing. The modules from the SNAP I/O line consist of digital, analog and special purpose I/O devices. The digital modules can sense or switch either AC or DC signals. Each digital module consists of four channels of similarly functional capability. The analog modules may be either two or four channels of calibrated that have a configurable variable signal level. There are special purpose modules such as serial and PID devices, but these have not been necessary in our systems.

In the System 21 product, the software in the M21 interfaces directly to the modules. There has been no requirement to use either a Brain or industrial controller from the SNAP line to read, write or control to the modules. The fastest controlled signals required are digital outputs, such as a pulse, and these have been controlled by the software.  The minimum pulse width possible is on the order of one millisecond. Typically, the timed outputs or pulses are used for motorized potentiometers. Analog inputs are used for measuring some voltage such as heaters, line speed, temperatures, voltage or other slowly varying process points. Their use was never intended as another high speed input such as for a sensor over the web or sheet. Interfacing to the SNAP modules is accomplished by two methods. The first is a serial connection that can communicate to each module individually and is the only choice for reading and writing to the analog modules. The second method uses a parallel cable to directly interface to each digital channel.

The SNAP I/O code related to the serial interface is concentrated in two areas. The first section is the code that interfaces to the SNAP I/O. This is primarily located in the file Eum_Ini2.cxx in the Linux system and Eum_Ini.cxx for pSOS systems. The header file that contains the structures are declared in Eum_Ini.h. The code has been modified slightly from the original pSOS version in the areas directly related to handling the serial interface in a Linux OS environment. This wasn’t solely due to the differences between the two operating systems in interfacing to serial I/O. The board itself forced some additional code to handle the disabling of the transmitter when receiving and vice-versa as the SNAP requires a two wire RS-485 serial line. The board probably would have worked ideally in a four-wire RS-485 configuration. The Tx/Rx switching was performed in the hardware automatically by a driver chip on the M21. Also, on the Linux system, there are three kernel device driver files and two user mode driver interface files library.c and board.c that interface to the Apex PC-104 boards – Stratus and Tracer. Finally, there are a number of header files associated with these files too.

There are a number of GCT Function Blocks (FB) under the Frame program in all Scanner GCT resources with the top level FB named Process1. This FB comprises all the GCT code for dealing with this low speed I/O. Digitals that control anything from Remote Off-sheet, Rollcut, Splice, Sheet Break, and more are made available to the scanner system internal code and are ‘wired’ to other FBs via Var References. The Process M21 will perform the same as a Scanner M21 in regards to interfacing to a rack of SNAP I/O modules along with having the same GCT FBs.

The vital statistics on the SNAP I/O as it exists now is shown in the table below. Typical configurations use a four-module board to fit in the end of the frame, but larger boards are available to accommodate more modules. Keep in mind that the maximums in the table are sometimes only possible at the cost of not having another module type. Also, on the current Linux system, due to the configuration requirements of the Tracer board, digital input or output module must be grouped by eights. Thus, one or two digital input modules may go in the first two slots and then one or two digital output modules would go into the third and fourth slots. This is not really a problem just different from the System 21 configuration requirements.

| Description | ## Maximum | ## Label |
| --- | --- | --- |
| Total Modules | 8 | MAX_MODULES |
| Analog Modules | 8 | MAX_ANALOG_MODULES |
| Digital Modules | 6 | MAX_DIGITAL_MODULES |
| Analogs per module | 2 | ANALOGS_PER_MODULE |
| Digitals per module | 4 | DIGITALS_PER_MODULE |
| Total Analogs (with no digitals) | 16 | MAX_ANALOGS |
| Total Digitals (with no analogs) | 24 | MAX_DIGITALS |

# Initialization

The start up of a typical scanner system consist first of creating and starting the three queues used by two tasks created by both the Eum_Ini2.cxx code and code from the GCT task. Once these queues are started, two tasks are created and started that will interface to the SNAP I/O hardware. These two tasks will sometimes be referred to as the SNAP Interface in this document. Similar to the M21 system, the current Linux based system accesses the SNAP I/O modules in two different ways. The first method is via a wire connection that has always been a RS-485 serial interface. Using this method, the analog channels are read and written via short serial packets. The baud rate is on the order of 9600 bps. The second method is reserved for reading and writing the digitals via a (somewhat) directly wired connection to the digital modules. On the M21, this connection was possible through multiple Zilog Z08536 CIO Counter/Timer and Parallel I/O chips. On the GesPac board, there is a PC-104 daughter board by Apex, called Tracer, which performs in a similar fashion. It has been additionally programmed to provide some more functionality that was previously on the M21’s Scanner Transition board, but that is not related to this document.

### Queues

The startup code in Eum_Ini2.cxx first creates and starts three pSOS style queues.

- The first queue is named "SNPI" and is accessed by the queue variable name 'qid_Snap_In'. This queue is dedicated to the passing of analog and digital parameters from GCT to one of the tasks that will interface to the SNAP I/O modules. The data to be output is passed from GCT to the interface in the snapStruct data structure presented later.

- The second queue is named "SNPO" and is accessed by the queue variable name 'qid_Snap_Out''. This queue is for transferring data to the GCT FBs from the SNAP Interfacing tasks. Similarly, the data is passed to GCT from the interface in the same snapStruct data structure presented later.

- The third queue is named "SNPC" and is accessed by the queue variable name 'qid_Snap_Ctl'. This queue is the output queue for the control outputs. This queue passes data that controls the duration of timed digital outputs’ active states. The two SNAP I/O interfacing tasks pass a short array containing the channel and time information for controlling the active duration.

### Tasks

Two tasks are created named "SNAP" and "SCTL" whose main software routine (program) names are Snap_Queue() and Snap_Ctl() respectively. These two tasks are tied closely together.

The Snap_Queue task is responsible for reading all inputs from the Snap I/O analog and digital modules. The data read in placed into the structure  snapStruct named ‘snapLocal’. This structure is then entered into the queue ‘qid_Snap_Out’ and passed to the GCT. In addition, analog and digital outputs are controlled by this task. These commands arrive from the GCT via the ‘qid_Snap_In’ queue and are output to the SNAP I/O modules. Timed digital outputs are initially activated by this task. Once a timed digital output is set, a message containing the channel and its duration is entering into the 'qid_Snap_Ctl' queue for the second task to process. The Snap_Ctl task handles the deactivation of all timed based digital outputs. Its resolution is on the order of one millisecond.

### Structures

The main data structure in the SNAP I/O system is the snap_struct structure presented below.

typedef struct snap_struct

{

unsigned int 	cDebug;              			// debugging level – Not used anymore

char  						cEnable;               	// true if to enable block, not used

char  						cDigIn[MAX_DIGITALS];  	// digital inputs

unsigned long cDigTim[MAX_DIGITALS];		// timed outputs

unsigned int  cAnaIn[MAX_ANALOGS];  		// analog inputs

unsigned long cCfgIn[MAX_MODULES];    // true if to reset configs

char  						cReset;  												// true to reset configurations

char  						cRstChg;                // true if to clear change flags

char  						cDigOut[MAX_DIGITALS]; 	// digital outputs

char  						cChgOut[MAX_DIGITALS]; 	// change flags

unsigned int  cAnaOut[MAX_ANALOGS];  	// analog outputs

unsigned long cCfgOut[MAX_MODULES];  	// config flags

unsigned long cQRcvErr;											// error counter

unsigned long cQSndErr;											// error counter

} snapStruct ;

# Operation

This section will discuss each task and what actions they perform in the running system.

### Snap_Queue Task

The Snap_Queue function controls almost all of the I/O to the SNAP I/O modules. Its series of actions are as follows:

- Extract any output data from the ‘qid_Snap_In’ queue into the msgbuf structure. This structure contains both analog and digital data to be written out. The data taken out goes into a local ‘snapLocal’ structure in the cDigIn and cDigTim arrays for digital outputs and cAnaIn array for analog outputs. Yes, that’s what it said in the code.

- Output any analog outputs that have changed.

- Determine if there are any digital outputs to be written.

- If there are and if any are of timed duration, send timed digital output data that is comprised of both channel and time duration information to ‘qid_Snap_Ctl’ queue in small concise arrays.

- Read digital channels to determine the current state, modify changed digital outputs and then write back the data to the digital channels. The Tracer board is connected to the SNAP modules via a parallel ribbon cable. All digital channels are read and written at once using Opto_read() and Opto_write() routines.

- Read analog inputs serially, placing the data values into cAnaOut array that is in the ‘snapLocal’ structure.

- Read all digital inputs using the Tracer board via the Opto_read() routine. Data is placed into the cDigOut array that is in the ‘snapLocal’ structure.

- Send the ‘snapLocal’ data structure to the GCT task via the ‘qid_Snap_Out’ queue.

- Wait 10 ms and repeat above steps.

### Snap_Ctl Task

The Snap_Ctl function is responsible for controlling timed digital outputs for such devices as motorized potentiometers or other timed outputs.

- Extract all messages from the ‘qid_Snap_Ctl’ queue should new messages have arrived since the last time checked.

- Update timing.

- Read current digital values from the Tracer board using the Opto_read() routine.

- Determine the shortest remaining time for any of the currently active digital outputs or two hundred milliseconds, whatever is the shortest time (delta). All timed digital output channels have a time period associated that indicates how long the output should be active. This time duration is decremented by this delta time. When a channel has timed out, its output value is reset or changed back to its inactive state.

- Determine what if any digital outputs require to be turned off due to the time duration having been completed..

- Write digital outputs back out.

- Sleep for the delta time period.

- Repeat above steps.

The data flow for SNAP I/O is shown below:

SNAP I/O Data Flow

AO

DO

qid_Snap_In

Snap_Queue()

Snap

I/O

Mod.

serial

AO

DO

AI

DI

qid_Snap_Out

GCT

Opto R/W

Tracer Board

Parallel

AI

DI

tDO

Opto R/W

qid_Snap_Ctl

Snap_Ctl()

tDO

### GCT

GCT uses the SNAP I/O to link the software to the outside environment. Digitals such as an Offsheet command initiated from the touchscreen interface, or because of a conditional being triggered, may be directed to the external world via the SNAP I/O interface. MD control typically may require pulsed outputs to turn motorized potentiometers. Some Linespeeds acquired via the SNAP I/O interface make their way into the software this way. Overall, all non-time critical I/O that can be described as not real-time I/O travels via the GCT/SNAP path.

The top level FB is called Process, however, typically in GCT OOP fashion, its instance will be named Process1(). This FB has a rather large number of variables and FBs within it. However, if you look at it from the standpoint of being just a group of above five FBs that have been cloned and repeated six to eight times each, then its not that bad. The actions performed in the Process1 FB may be broken and followed if just one I/O module type is followed through the code at a time. As you will see later, the data is sometimes moved/copied for the purpose of  packing/unpacking and also emulating the modules on the SNAP I/O.

The first to be looked at is an analog output in which the goal is to take a floating point value in GCT, convert to a integer and pass this to the SNAP I/O interface. The first FB in Process1() to be discussed is called GetPOs() whose main purpose for the analog value is to select the method of conversion and scaling of the data. The value is a Var Reference to another variable elsewhere in the GCT software. Three choices are possible and thus are selected by Booleans that have tuned into via the AIF and VRF files. The three choices and there actions are listed below:

IF AbsDevN THEN

AON := AONRef - DevTgtNRef ;

ELSIF PctDevN THEN

AON := (AONRef - DevTgtNRef) / DevTgtNRef ;

ELSE

AON := AONRef ;

END_IF ;

Where: (N may be 0 or 1 similar to the two analogs on the module.)

If AbsDevN is TRUE, then AON = (AO variable) – (Offset value)

or

If PctDevN is TRUE, then AON = [(AO variable) – (Scale factor)] / (Scale factor)

or

Just AON = (AO variable)

These values AO0 and AO1 are passed into AnaOut FB where they are first clamped to stay within a low and high range. Next, the data will be scaled to a digital value using the simple formula:

IF valueN < low0 THEN

valueN := low0 ;

END_IF ;

IF valueN > high0 THEN

valueN := high0 ;

END_IF ;

aOut[N] := REAL_TO_UINT(IN:=(valueN - offsetN)/gainN);

The SNAP I/O analog output modules typically are 12 bit DAC and thus have a digital range from 0 to 4095 counts. The final action in the FB is to clamp the digital values to the module’s range or maybe restrict it to a user specified range. For example, it may be clamped due to that whatever the output is connected to, it only accepts 0 to 5 volts while the module has a 0 to 10 volt range. This code is as follows:

IF aOut[N] < lowOut THEN

aOut[N] := lowOut ;

END_IF ;

IF aOut[N] > highOut THEN

aOut[N] := highOut ;

END_IF ;

Next, the data is copied and packaged prior to being sent to the SNAP Interface. The array is passed through into SnapFB1(), through AnaSw1() where it is moved into another array and passed to SnapIO1(). In SnapIO1(), the data is copied into the local snapStruct structure variable ‘snapStructArea’ . Once the data has been copied into the data structure along with digital outputs that are discussed next, the data structure is placed into the ‘qid_Snap_In’ queue and passed over to the SNAP Interface.

The next module covered is the digital output module. This is output also handles pulsed outputs. Once again, the first FB examined is GetPOs() whose main purpose for the digital value is to select the source of the data. In this case, there are just two choices. The digital state may be referenced from elsewhere in GCT or be written into the DONWrite variable remotely.

IF DONByRef THEN

DON := DONRef ;

ELSE

DON := DONWrite ;

END_IF ;

(N may range from 0 to 3, which is the number of channels on a digital module.)

Another variable is a Var Reference (TimOutNRef)  that points to variables in other GCT FBs such as :

MDControl1.LSControl.outNPls

MDControl1.LSControl.outPPls

MDControl1.SSControl.outNPls

MDControl1.SSControl.outPPls

The DOs and TimOuts are wired into the next FB, DigOut() where they are copied into arrays equivalent to a digital module in that the array size is four elements. From here, the arrays make their way to the next FB SnapFB() to DigSW() where they are copied into array equivalent to the maximum number of digitals (24) that the SNAP Interface on a System 21 may handle. Finally, the array makes its way over to the SnapIO1() FB. Here, similar to the analog outputs, it is moved into the snapStruct structure ‘snapStructArea’.

Analog and digital inputs arrive in GCT via the ‘qid_Snap_Out’ queue. The data is once again in the snapStruct structure ‘snapStructArea’. The data is extracted from the data structure and moved into GCT variable arrays. For digital outputs, the array is 24 elements, and for analog outputs, the arrays is 16 elements. These arrays are passed through AnaSw1() and sub divided into smaller arrays that are equivalent to the number of channels on their respective modules.

The analog array is passed over to AnaIn00() for conversion from digital (integer) to a floating point number. Tests are performed to prevent drastic changes of more than half scale at a time. Otherwise, the algorithm is shown below. The resulting variable anaN has now been converted to a floating point number.

Temp := (gainN * UINT_TO_REAL(IN := aValue[N])) + offsetN ;

aNCtr := 0 ;

IF Init0 THEN

anaN := Temp ;

InitN := FALSE ;

ELSE

anaN := (anaN * ANFF) + ((1.0 - ANFF) * Temp);

END_IF ;

(N may range from 0 to 1, which is the number of channels on an analog module.)

Where:

ANFF is usually set to 0.5.

Digital inputs follow a similar path where the arrays are broken down to the same size as the number of digital inputs (4) on a SNAP module, then passed through another FB to break it down into individual Boolean variables. The first digital on each channel can be configured to latch on changes to capture short pulses. The output of the FB is a Boolean that is set to true should even a momentary pulse that occurred between executions of the GCT FB.

# Tuning

??